DEVELOPER-READY IMPLEMENTATION DOCUMENT

Terrain-Aware Autonomous Rover

Everything needed to start coding: wiring, module structure, formulas, constants, and pseudocode.

Companion to: Design Requirements Document v1.0

Team: 2 × EEE · 2 × CSE — MBCET

# 1. Architecture at a Glance

Two layers, connected over WiFi/MQTT:

- EE layer (on the rover): MPU6050 → ESP32 → L298N → 2× DC motors

- CS layer (laptop/Pi during development, optional once inference moves fully on-device): MQTT ingest → windowing → feature extraction → classifier → smoothing → speed policy → dashboard

Target end-state: the classifier and control loop run entirely on the ESP32 (Section 5 covers exporting the trained model to C++). The laptop-side pipeline is used during development, data collection, and training, and optionally kept for the live dashboard.

## 1.1 Repository layout

/firmware ESP32 C++ (Arduino/ESP-IDF) — sensing, control loop, on-device inference /ml Python — feature extraction, training, model export /dashboard Flask/Streamlit — live label, confidence, trace, speed /data Collected + labelled vibration sessions (raw + processed) /docs DRD, this document, test track photos, results

# 2. Hardware Wiring

## 2.1 Pin map (ESP32 DevKit V1)

| Signal | ESP32 Pin | Connects To | Notes |
| --- | --- | --- | --- |
| I2C SDA | GPIO 21 | MPU6050 SDA | Shared I2C bus |
| I2C SCL | GPIO 22 | MPU6050 SCL | Shared I2C bus |
| Right motor PWM | GPIO 25 (ENA) | L298N ENA | PWM speed, right side |
| Right motor IN1 | GPIO 26 | L298N IN1 | Direction |
| Right motor IN2 | GPIO 27 | L298N IN2 | Direction |
| Left motor PWM | GPIO 14 (ENB) | L298N ENB | PWM speed, left side |
| Left motor IN3 | GPIO 12 | L298N IN3 | Direction |
| Left motor IN4 | GPIO 13 | L298N IN4 | Direction |
| MPU6050 power | 3.3V / GND | MPU6050 VCC/GND | — |
| L298N logic power | 5V (VIN) / GND | L298N +5V / GND | Motor supply from battery pack, not ESP32 5V pin |

Mounting note: bolt the MPU6050 rigidly to the chassis frame, near a wheel mount — not on foam tape or a loose breadboard. Compliant mounting low-pass-filters the signal and is the single biggest driver of classification accuracy.

# 3. Firmware Module (ESP32 / C++)

## 3.1 Sampling loop

Sample the IMU at 100 Hz on a fixed timer. Maintain a circular buffer of the last 100 samples (1 second of data).

const int WINDOW_SIZE = 100; // 1s @ 100Hz float window[WINDOW_SIZE]; int windowIdx = 0; const unsigned long SAMPLE_PERIOD_MS = 10; // 100 Hz void loop() { if (millis() - lastSample >= SAMPLE_PERIOD_MS) { lastSample = millis(); float z = readAccelZ(); // from MPU6050 window[windowIdx % WINDOW_SIZE] = z; windowIdx++; if (windowIdx >= WINDOW_SIZE) runInferenceCycle(); } }

## 3.2 Feature extraction (5-D vector)

Computed once per window (every ~10 new samples for 50% overlap in the training pipeline; on-device this can run every window for simplicity).

| Feature | Formula | Why it matters |
| --- | --- | --- |
| std | standard deviation of the window | Primary discriminator — vibration intensity |
| rms | sqrt(mean(x²)) | Overall energy, orientation-independent |
| peak | max(¦x¦) / rms | Impulsiveness — separates spiky gravel from smooth carpet |
| p2p | max(x) − min(x) | Range of motion within the window |
| zcr | count of sign changes between consecutive samples | Cheap proxy for dominant frequency |

void extractFeatures(float *buf, int n, float *out5) { float mean = 0, sq = 0, mx = buf[0], mn = buf[0], rmsAcc = 0; for (int i = 0; i < n; i++) { mean += buf[i]; rmsAcc += buf[i] * buf[i]; if (buf[i] > mx) mx = buf[i]; if (buf[i] < mn) mn = buf[i]; } mean /= n; float rms = sqrt(rmsAcc / n); for (int i = 0; i < n; i++) sq += (buf[i]-mean)*(buf[i]-mean); float std = sqrt(sq / n); float peak = rms > 0.001 ? max(fabs(mx), fabs(mn)) / rms : 0; int zcr = 0; for (int i = 0; i < n-1; i++) if (buf[i]*buf[i+1] < 0) zcr++; out5[0]=std; out5[1]=peak; out5[2]=rms; out5[3]=mx-mn; out5[4]=(float)zcr; }

## 3.3 Classifier integration

The trained scikit-learn decision tree is exported to a C++ header (see Section 5.3) and included directly in firmware:

#include "terrain_classifier.h" Eloquent::ML::Port::DecisionTree clf; const char* label = clf.predictLabel(features); // "tile" ¦ "mat" ¦ "carpet" ¦ "gravel"

Do not hand-edit this header. Regenerate it whenever the model is retrained (Section 5.4).

## 3.4 Smoothing (majority vote)

const int VOTE_WINDOW = 5; char* voteBuf[VOTE_WINDOW]; int voteIdx = 0; voteBuf[voteIdx % VOTE_WINDOW] = label; voteIdx++; char* smoothedLabel = majorityOf(voteBuf, min(voteIdx, VOTE_WINDOW));

## 3.5 Driving profile lookup

Each smoothed label maps to a full driving profile — not just a speed value. Acceleration ramp is the commanded proxy for torque (no current sensor in this build).

| Terrain | PWM (0–255) | Accel. ramp | Turn gain |
| --- | --- | --- | --- |
| Tile | 220 | 0.2 s | 1.0 |
| Mat | 190 | 0.3 s | 0.9 |
| Carpet | 170 | 0.4 s | 0.8 |
| Gravel | 130 | 0.6 s | 0.6 |

struct Profile { int pwm; float rampSec; float turnGain; }; Profile PROFILES[4] = { {220, 0.2, 1.0}, // tile {190, 0.3, 0.9}, // mat {170, 0.4, 0.8}, // carpet {130, 0.6, 0.6}, // gravel }; // apply via slew-rate-limited PWM ramp toward PROFILES[i].pwm // scale left/right differential by PROFILES[i].turnGain when turning

## 3.6 Slope-handling state machine

Pitch is computed from the accelerometer's gravity vector: pitch = atan2(-ax, sqrt(ay² + az²)).

| State | Entry Condition | Action | Exit → Next State |
| --- | --- | --- | --- |
| NORMAL | default | Apply terrain-classified profile (3.5) | pitch > 6° → DETECT |
| DETECT | pitch > 6° | Hold heading, sample pitch briefly | pitch < 12° → CLIMB pitch ≥ 12° → RETREAT |
| RETREAT | steep slope confirmed | Reverse ~15–20 cm | distance reached → APPROACH |
| APPROACH | after retreat | Fastest accel. ramp, max PWM, turn gain locked low | pitch confirms incline → CLIMB |
| CLIMB | on incline (mild or steep path) | Elevated PWM, minimal turning | pitch < 2° → NORMAL |

enum SlopeState { NORMAL, DETECT, RETREAT, APPROACH, CLIMB }; SlopeState state = NORMAL; const float DETECT_DEG = 6.0, STEEP_DEG = 12.0, FLAT_DEG = 2.0; const float RETREAT_CM = 17.5; // midpoint of 15-20cm range, tune on hardware

## 3.7 MQTT topics

| Topic | Direction | Payload |
| --- | --- | --- |
| rover/imu | ESP32 → laptop | Raw/windowed accel+gyro samples (dev/training only) |
| rover/state | ESP32 → laptop | Current label, confidence, PWM, slope state — feeds dashboard |
| rover/cmd | laptop → ESP32 | Override PWM set-point (used only if inference is off-device) |

# 4. ML Training Pipeline (Python)

## 4.1 Data collection protocol

- 4 classes: tile, mat, carpet, gravel

- 4 × 3-minute sessions per class, on separate days — not one long run

- 3 PWM levels (low/med/high) per class, so the model learns terrain, not speed

- Split by session, not by window — train on sessions 1–3, hold out session 4 (prevents inflated accuracy from overlapping windows leaking into test)

## 4.2 Feature extraction (Python, mirrors Section 3.2)

def extract_features(sig): std = sig.std() rms = np.sqrt((sig**2).mean()) peak = np.abs(sig).max() / rms if rms > 1e-3 else 0 p2p = sig.max() - sig.min() zcr = ((sig[:-1] * sig[1:]) < 0).sum() return [std, peak, rms, p2p, zcr]

## 4.3 Model selection

- Start with DecisionTreeClassifier(max_depth=5) — interpretable, prints as a readable tree

- Compare against KNN (k=5) and RandomForest (100 trees) via session-wise cross-validation

- Pick the simplest model within ~2% of the best accuracy

- Report a confusion matrix — expect carpet/mat to be the most confusable pair

## 4.4 Exporting to C++

Use micromlgen (or scikit-learn's own tree export, hand-adapted) to convert the trained tree into the header used in Section 3.3:

from micromlgen import port cpp_code = port(clf, classmap={0:'tile', 1:'mat', 2:'carpet', 3:'gravel'}) with open('terrain_classifier.h', 'w') as f: f.write(cpp_code)

Regenerate and replace firmware/terrain_classifier.h every time the model is retrained on new data. Do not hand-edit the generated header.

# 5. Dashboard Module

Subscribes to rover/state over MQTT. Minimum required views:

- Live accelerometer trace (scrolling, last ~3 seconds)

- Current predicted label + confidence

- Current PWM / speed and active driving profile

- Slope state indicator (NORMAL / DETECT / RETREAT / APPROACH / CLIMB)

Suggested stack: Flask + Chart.js, or Streamlit for faster iteration. Update rate: match MQTT message rate, no additional polling needed.

# 6. Constants Quick Reference

| Constant | Value | Where used |
| --- | --- | --- |
| Sample rate | 100 Hz | Firmware sampling loop (3.1) |
| Window size / overlap | 1 s / 50% | Feature extraction (3.2, 4.2) |
| Vote window | 5 predictions | Smoothing (3.4) |
| Tree depth | 5 | Model training (4.3) |
| Slope detect threshold | ~6° pitch | Slope state machine (3.6) |
| Slope steep threshold | ~12° pitch | Slope state machine (3.6) |
| Slope flat (resume) threshold | ~2° pitch | Slope state machine (3.6) |
| Retreat distance | 15–20 cm | RETREAT state (3.6) |
| PWM range | 130–220 (of 255) | Driving profile (3.5) |
| Accel. ramp range | 0.2–0.6 s | Driving profile (3.5) |

All thresholds above are initial estimates carried over from the design phase (see DRD, Section 5.2) and should be tuned once real hardware data is available — particularly the pitch thresholds and retreat distance, which were never measured on a physical slope.

Next steps for whoever picks this up: (1) wire hardware per Section 2, (2) flash a minimal firmware that just streams IMU data over MQTT, (3) run the data collection protocol (4.1), (4) train + export the classifier (4.3–4.4), (5) integrate the generated header and driving profile into firmware, (6) bring up the dashboard, (7) validate against the DRD's acceptance criteria.